# ESP32 S3 DHT22 Data Collection and Real Time Server Guide

Arduino firmware  Node.js API  SQLite storage  Live dashboard

Prepared for Karan Sambhaji Jadhav

ESP32 S3 WROOM 1 N16R8 and DHT22 module

29 September 2026

# Purpose and Result

This guide provides a complete local network implementation for collecting air temperature and humidity. The ESP32-S3 reads a DHT22 on GPIO4, labels each observation as actual or simulated, sends JSON to an Express API, stores the record in SQLite, and updates a browser graph without a page refresh. The included firmware can continue testing the server with simulated data while the physical DHT22 is being diagnosed.

Complete source files accompany this guide. Follow the sections in order for the first installation. The final verification is a new row in SQLite and a new point on the dashboard after each ESP32 POST request.

# Contents

1 System Architecture

2 Hardware and Software Requirements

3 DHT22 Wiring

4 Arduino IDE Preparation

5 Server Installation

6 ESP32 Firmware Configuration

7 Starting and Testing the Server

8 Uploading and Running the ESP32

9 API and Database Documentation

10 Real Time Dashboard

11 End to End Verification

12 Troubleshooting

13 Research Data Integrity

14 Security and Deployment

15 Source References

Appendix A ESP32 Firmware

Appendix B Node Server

Appendix C Dashboard

# 1 System Architecture

The ESP32-S3 acts as the data acquisition device. It reads the DHT22 or generates a labelled simulated observation, connects to 2.4 GHz Wi-Fi, and sends an HTTP POST request. The Express server validates the JSON before writing to SQLite. After insertion, the server broadcasts the stored row through Server-Sent Events. The dashboard receives the event and updates the graph.

Figure 1  End to end sensor data flow

| Component | Responsibility | Output |
| --- | --- | --- |
| DHT22 | Measures ambient air temperature and relative humidity | Digital pulse sequence |
| ESP32-S3 | Reads or simulates data and performs the HTTP POST | JSON request |
| Express server | Validates requests and exposes API routes | HTTP responses and SSE events |
| SQLite | Stores durable timestamped observations | sensor_readings rows |
| Dashboard | Loads history and plots new events | Cards graph and table |

# 2 Hardware and Software Requirements

| Item | Purpose | Notes |
| --- | --- | --- |
| ESP32-S3 Dev Module | Wi-Fi microcontroller | ESP32-S3-WROOM-1-N16R8 |
| DHT22 three-pin module | Temperature and humidity | Pins must be identified by labels |
| Three jumper wires | VCC DATA and GND | Use short secure wires |
| USB-C data cable | Power programming and Serial Monitor | Charge-only cables cannot upload |
| Computer or server | Runs Node.js API and dashboard | Windows or Ubuntu |
| 2.4 GHz Wi-Fi | Local ESP32 connectivity | ESP32-S3 does not join 5 GHz-only networks |

## Software

Arduino IDE with the esp32 package by Espressif Systems

DHT sensor library by Adafruit

Adafruit Unified Sensor library

Node.js 20 or later with npm

Express and better-sqlite3 installed by npm

A modern browser with JavaScript enabled

# 3 DHT22 Wiring

Disconnect USB power before changing wires. Use the labels printed beside the module pins because different three-pin boards can use different physical orders.

| DHT22 label | ESP32-S3 pin | Function |
| --- | --- | --- |
| VCC or plus | 3V3 | Sensor supply |
| OUT DATA or S | GPIO4 | Digital sensor data |
| GND or minus | GND | Common electrical reference |

A three-pin DHT22 module normally includes a pull-up resistor between VCC and OUT. Resistance of approximately 4.7 kΩ or 10 kΩ is normal. A direct resistance near 0 Ω is not normal. Measure resistance only while the module is disconnected and unpowered.

In the firmware, DHT_PIN 4 means ESP32 GPIO4. It does not mean the fourth physical sensor pin or the fourth position on the ESP32 header.

# 4 Arduino IDE Preparation

## 4.1 Install ESP32 board support

1. Open Arduino IDE and select File then Preferences.

2. Add https://espressif.github.io/arduino-esp32/package_esp32_index.json to Additional Boards Manager URLs.

3. Open Boards Manager, search for esp32, and install esp32 by Espressif Systems.

4. Select Tools then Board then ESP32S3 Dev Module.

5. Connect the board through the USB-C connector marked COM or UART and select its COM port.

## 4.2 Select board settings

| Setting | Value |
| --- | --- |
| Board | ESP32S3 Dev Module |
| CPU Frequency | 240 MHz |
| Flash Size | 16 MB or 128 Mb |
| Flash Mode | QIO 80 MHz |
| PSRAM | OPI PSRAM |
| USB CDC On Boot | Disabled when using the COM or UART connector |
| Upload Speed | 460800 or 115200 if uploads are unstable |

## 4.3 Install sensor libraries

1. Open Tools then Manage Libraries.

2. Install DHT sensor library by Adafruit.

3. Install Adafruit Unified Sensor.

4. Remove incompatible duplicate DHT libraries if Arduino reports multiple libraries for DHT.h.

# 5 Server Installation

Copy the server directory from the project bundle to the computer that will receive readings.

## 5.1 Confirm Node.js

node --version

npm --version

Install a current Node.js LTS release if Node.js is missing or older than version 20.

## 5.2 Install dependencies

Open PowerShell or a terminal inside the server directory and run:

npm install

npm reads package.json and installs Express and better-sqlite3. The sensor-data.db file is created automatically when the server first starts.

## 5.3 Find the server address

On Windows run:

ipconfig

On Ubuntu run:

hostname -I

Use the active local IPv4 address, for example 192.168.1.100. Do not put localhost or 127.0.0.1 in the ESP32 firmware because those addresses would refer to the ESP32 itself.

# 6 ESP32 Firmware Configuration

Open esp32_dht22_client.ino and replace the configuration values near the top:

const char* WIFI_NAME = "YOUR_2_4_GHZ_WIFI_NAME";

const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

const char* SERVER_URL = "http://192.168.1.100:3000/api/readings";

## 6.1 Select the data mode

| Mode | Behaviour | Recommended use |
| --- | --- | --- |
| SIMULATED_ONLY | Always generates labelled test observations | Initial API and graph testing |
| DHT22_ONLY | Sends only successful GPIO4 readings | Final physical data collection |
| DHT22 WITH FALLBACK | Uses DHT22 first and simulated data after failure | Continuing pipeline development while repairing the sensor |

The supplied file starts in fallback mode:

const DataMode DATA_MODE = DHT22_WITH_SIMULATED_FALLBACK;

Every record contains simulated and source fields. Keep these fields in research datasets so test observations can never be mistaken for measurements.

## 6.2 Reading interval

The default interval is ten seconds:

const unsigned long READING_INTERVAL_MS = 10000;

The DHT22 should not be sampled faster than once every two seconds. A ten-second interval is suitable for initial testing. Longer intervals such as one or five minutes are normally more appropriate for crop monitoring.

# 7 Starting and Testing the Server

## 7.1 Start the process

npm start

Expected output:

Sensor server running on port 3000

Dashboard: http://localhost:3000

## 7.2 Test the health endpoint

Open the following address on the server computer:

http://localhost:3000/api/health

Expected response:

{

"status": "healthy",

"dashboard_clients": 0,

"server_time": "2026-09-29T05:30:00.000Z"

}

## 7.3 Test from another device

From a phone or another computer on the same Wi-Fi, open:

http://192.168.1.100:3000/api/health

Replace the example address. If localhost works but the local IP does not, allow Node.js or inbound TCP port 3000 through the operating system firewall.

## 7.4 Test the POST route manually

PowerShell example:

$body = @{

device_id = "manual-test"

crop = "fenugreek_microgreens"

reading_number = 1

uptime_ms = 1000

air_temperature_c = 27.4

air_humidity_percent = 68.2

simulated = $true

source = "manual_test"

sensor_error = ""

} | ConvertTo-Json

Invoke-RestMethod -Method Post `

-Uri "http://localhost:3000/api/readings" `

-ContentType "application/json" `

-Body $body

# 8 Uploading and Running the ESP32

1. Start the Node.js server before powering the ESP32 client.

2. Open the included Arduino sketch and enter Wi-Fi and server settings.

3. Confirm ESP32S3 Dev Module and the correct COM port.

4. Click Verify and then Upload.

5. If upload remains at Connecting, hold BOOT, press and release RESET, release BOOT, and upload again.

6. Open Serial Monitor at 115200 baud and press RESET once.

A successful cycle resembles:

Wi-Fi connected

ESP32 IP address: 192.168.1.105

Attempting DHT22 reading from GPIO4

Using simulated fallback data

Sending JSON:

{...}

HTTP response code: 201

Server response: {"success":true,...}

## Status LED

| Colour | State |
| --- | --- |
| Blue | Connecting to Wi-Fi |
| Yellow | Collecting a reading |
| Purple | Sending an HTTP request |
| Green | Successful connection or POST |
| Orange | Server returned a non-success HTTP status |
| Red | Sensor network or request failure |

# 9 API and Database Documentation

| Method and route | Purpose | Response |
| --- | --- | --- |
| POST /api/readings | Validate and store one observation | 201 success or 400 validation error |
| GET /api/readings?limit=60 | Return recent observations | JSON array newest first |
| GET /api/events | Open real-time SSE stream | text/event-stream |
| GET /api/health | Check server availability | Health JSON |

## 9.1 JSON request contract

| Field | Type | Meaning |
| --- | --- | --- |
| device_id | String required | Stable identifier for the ESP32 |
| crop | String | Crop associated with the installation |
| reading_number | Integer | Sequence since the current ESP32 boot |
| uptime_ms | Integer | Milliseconds since ESP32 boot |
| air_temperature_c | Number required | Air temperature in Celsius |
| air_humidity_percent | Number required | Relative humidity from 0 to 100 |
| simulated | Boolean | True for generated or fallback data |
| source | String | simulated_test simulated_fallback or dht22_gpio4 |
| sensor_error | String | Physical sensor failure retained with fallback data |

## 9.2 SQLite schema

CREATE TABLE sensor_readings (

id INTEGER PRIMARY KEY AUTOINCREMENT,

device_id TEXT NOT NULL,

crop TEXT,

reading_number INTEGER,

device_uptime_ms INTEGER,

air_temperature_c REAL NOT NULL,

air_humidity_percent REAL NOT NULL,

simulated INTEGER NOT NULL DEFAULT 0,

source TEXT,

sensor_error TEXT,

received_at TEXT NOT NULL DEFAULT CURRENT_TIMESTAMP

);

SQLite stores simulated as 0 or 1. CURRENT_TIMESTAMP is UTC. The dashboard converts it to the browser's local time. WAL mode improves reliability when inserts and dashboard reads occur together.

# 10 Real Time Dashboard

Open the dashboard from the server:

http://localhost:3000

Or from another device on the same network:

http://192.168.1.100:3000

When the page opens, it requests recent records from GET /api/readings. It then opens EventSource on /api/events. After each database insert, the server sends the stored row to every connected browser. The dashboard appends temperature and humidity without polling or refreshing.

The red line uses the left temperature axis.

The blue line uses the right humidity axis from 0 to 100 percent.

The chart retains the latest 60 points.

The table retains the latest 10 points.

The source badge distinguishes actual and simulated observations.

Chart.js loads from jsDelivr. The viewing device needs internet access for the chart library unless Chart.js is downloaded and served locally.

# 11 End to End Verification

| Step | Check | Expected evidence |
| --- | --- | --- |
| 1 | Server starts | Port 3000 message appears |
| 2 | Health route opens | status healthy JSON |
| 3 | Dashboard opens | Cards graph and table are visible |
| 4 | ESP32 joins Wi-Fi | Local IP printed in Serial Monitor |
| 5 | ESP32 posts JSON | HTTP 201 printed |
| 6 | Server stores row | Reading stored appears in terminal |
| 7 | Dashboard receives event | New point appears without refresh |
| 8 | History persists | Point returns after server and browser restart |

# 12 Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| Wi-Fi connection failed | Wrong credentials or 5 GHz-only network | Use exact credentials and enable 2.4 GHz |
| DHT22 returns nan | Wrong pin order loose wire missing power or damaged sensor | Follow printed labels and verify 3.3 V and DATA |
| No sensor response | DATA is not connected to GPIO4 or sensor is unpowered | Check OUT to GPIO4 and common GND |
| DATA line stuck LOW | Short to ground or reversed wiring | Disconnect power and inspect wiring |
| HTTP response code -1 | Server unreachable or firewall blocked | Verify server IP port and firewall |
| HTTP 400 | Missing invalid or out-of-range JSON | Inspect the printed payload |
| HTTP 404 | Incorrect route | Use /api/readings |
| HTTP 201 but graph unchanged | Dashboard SSE disconnected or Chart.js unavailable | Check Live indicator browser console and internet |
| localhost works but LAN IP fails | Firewall or wrong network adapter | Allow TCP 3000 and use the active IPv4 address |
| COM port access denied | Serial Monitor or another program owns the port | Close all serial applications and reconnect |
| Upload stuck at Connecting | Board not in bootloader | Use BOOT and RESET sequence |
| Chart time appears wrong | UTC timestamp interpreted incorrectly | Retain the supplied UTC conversion function |

# 13 Research Data Integrity

Simulated observations are useful for testing software, but they are not experimental measurements. Retain simulated, source, sensor_error, device_id, reading_number, uptime, and received_at in every stored record. Exclude simulated equals 1 from model training, hypothesis testing, descriptive statistics, and reported agricultural results unless the analysis explicitly studies simulation.

Example query for actual sensor data only:

SELECT *

FROM sensor_readings

WHERE simulated = 0

ORDER BY received_at;

Example query for diagnosing fallback frequency:

SELECT source, sensor_error, COUNT(*) AS observations

FROM sensor_readings

GROUP BY source, sensor_error;

For longer studies, add experiment_id, plot_id, treatment_id, sensor_serial_number, calibration_date, firmware_version, and measurement_location. Use a stable clock source if precise sampling time is required because device_uptime_ms resets after every reboot.

# 14 Security and Deployment

The supplied implementation is intended for a trusted local network. Do not expose port 3000 directly to the public internet. Before remote deployment, add HTTPS, per-device authentication, request rate limits, payload logging limits, process supervision, database backups, and reverse-proxy configuration.

Store Wi-Fi passwords outside source control before publishing code.

Use an API token or signed device credential for every POST request.

Terminate HTTPS at a trusted reverse proxy and reject plain external HTTP.

Run the Node process as a non-root user.

Back up sensor-data.db and its WAL files consistently.

Use systemd or a container restart policy for automatic recovery.

# 15 Source References

1. Espressif Systems. Arduino ESP32 Wi-Fi API. https://docs.espressif.com/projects/arduino-esp32/en/latest/api/wifi.html

2. Espressif Systems. Arduino ESP32 HTTPClient examples. https://github.com/espressif/arduino-esp32/tree/master/libraries/HTTPClient/examples

3. Adafruit. DHT11 DHT22 and AM2302 Sensors. https://learn.adafruit.com/dht

4. Express. Serving static files in Express. https://expressjs.com/en/starter/static-files/

5. Chart.js. Getting Started and Line Charts. https://www.chartjs.org/docs/latest/

6. MDN Web Docs. EventSource and Server-Sent Events. https://developer.mozilla.org/en-US/docs/Web/API/EventSource

7. WiseLibs. better-sqlite3. https://github.com/WiseLibs/better-sqlite3

# Appendix A ESP32 Firmware

File: esp32/esp32_dht22_client.ino. Replace Wi-Fi credentials and the server address before uploading.

#include <Arduino.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <DHT.h>

#define DHT_PIN 4

#define DHT_TYPE DHT22

#define RGB_LED_PIN 48

// Replace these three values before uploading.

const char* WIFI_NAME = "YOUR_2_4_GHZ_WIFI_NAME";

const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

const char* SERVER_URL = "http://192.168.1.100:3000/api/readings";

const char* DEVICE_ID = "esp32s3-fenugreek-01";

const char* CROP_NAME = "fenugreek_microgreens";

DHT dht(DHT_PIN, DHT_TYPE);

enum DataMode {

SIMULATED_ONLY,

DHT22_ONLY,

DHT22_WITH_SIMULATED_FALLBACK

};

// Start with fallback mode while diagnosing the sensor.

// Change to DHT22_ONLY after the physical sensor is reliable.

const DataMode DATA_MODE = DHT22_WITH_SIMULATED_FALLBACK;

struct SensorReading {

float temperature;

float humidity;

bool valid;

bool simulated;

String source;

String error;

};

const unsigned long READING_INTERVAL_MS = 10000;

unsigned long previousReadingTime = 0;

unsigned long readingNumber = 0;

float simulatedTemperature = 27.0;

float simulatedHumidity = 68.0;

void setLed(uint8_t red, uint8_t green, uint8_t blue) {

rgbLedWrite(RGB_LED_PIN, red, green, blue);

}

SensorReading generateSimulatedReading() {

SensorReading reading;

simulatedTemperature += random(-25, 26) / 100.0;

simulatedHumidity += random(-60, 61) / 100.0;

simulatedTemperature = constrain(simulatedTemperature, 20.0, 40.0);

simulatedHumidity = constrain(simulatedHumidity, 30.0, 95.0);

reading.temperature = simulatedTemperature;

reading.humidity = simulatedHumidity;

reading.valid = true;

reading.simulated = true;

reading.source = "simulated_test";

reading.error = "";

return reading;

}

SensorReading readDHT22Reading() {

SensorReading reading;

reading.temperature = NAN;

reading.humidity = NAN;

reading.valid = false;

reading.simulated = false;

reading.source = "dht22_gpio4";

reading.error = "";

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

if (isnan(temperature) || isnan(humidity)) {

reading.error = "DHT22 communication failed";

return reading;

}

if (temperature < -40.0 || temperature > 80.0) {

reading.error = "Temperature outside DHT22 range";

return reading;

}

if (humidity < 0.0 || humidity > 100.0) {

reading.error = "Humidity outside valid range";

return reading;

}

reading.temperature = temperature;

reading.humidity = humidity;

reading.valid = true;

return reading;

}

SensorReading collectReading() {

if (DATA_MODE == SIMULATED_ONLY) {

Serial.println("Mode: simulated data");

return generateSimulatedReading();

}

Serial.println("Attempting DHT22 reading from GPIO4");

SensorReading actualReading = readDHT22Reading();

if (actualReading.valid) {

Serial.println("DHT22 reading successful");

return actualReading;

}

Serial.print("DHT22 error: ");

Serial.println(actualReading.error);

if (DATA_MODE == DHT22_ONLY) {

return actualReading;

}

Serial.println("Using simulated fallback data");

SensorReading fallback = generateSimulatedReading();

fallback.source = "simulated_fallback";

fallback.error = actualReading.error;

return fallback;

}

bool connectToWiFi() {

if (WiFi.status() == WL_CONNECTED) {

return true;

}

setLed(0, 0, 20); // Blue

Serial.print("Connecting to Wi-Fi");

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_NAME, WIFI_PASSWORD);

unsigned long startedAt = millis();

while (

WiFi.status() != WL_CONNECTED &&

millis() - startedAt < 20000

) {

Serial.print(".");

delay(500);

}

Serial.println();

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Wi-Fi connection failed");

setLed(20, 0, 0); // Red

return false;

}

Serial.println("Wi-Fi connected");

Serial.print("ESP32 IP address: ");

Serial.println(WiFi.localIP());

setLed(0, 20, 0); // Green

return true;

}

String createJson(const SensorReading& reading) {

readingNumber++;

String json;

json.reserve(350);

json += "{";

json += "\"device_id\":\"" + String(DEVICE_ID) + "\",";

json += "\"crop\":\"" + String(CROP_NAME) + "\",";

json += "\"reading_number\":" + String(readingNumber) + ",";

json += "\"uptime_ms\":" + String(millis()) + ",";

json += "\"air_temperature_c\":" + String(reading.temperature, 2) + ",";

json += "\"air_humidity_percent\":" + String(reading.humidity, 2) + ",";

json += "\"simulated\":";

json += reading.simulated ? "true" : "false";

json += ",";

json += "\"source\":\"" + reading.source + "\",";

json += "\"sensor_error\":\"" + reading.error + "\"";

json += "}";

return json;

}

bool postReading(const SensorReading& reading) {

if (!reading.valid) {

Serial.println("Invalid reading. POST skipped.");

return false;

}

if (!connectToWiFi()) {

Serial.println("No Wi-Fi. POST skipped.");

return false;

}

String json = createJson(reading);

Serial.println("Sending JSON:");

Serial.println(json);

setLed(20, 0, 20); // Purple

WiFiClient client;

HTTPClient http;

http.setConnectTimeout(5000);

http.setTimeout(5000);

if (!http.begin(client, SERVER_URL)) {

Serial.println("Unable to initialize HTTP connection");

setLed(20, 0, 0);

return false;

}

http.addHeader("Content-Type", "application/json");

int responseCode = http.POST(json);

if (responseCode <= 0) {

Serial.print("HTTP request failed: ");

Serial.println(http.errorToString(responseCode));

http.end();

setLed(20, 0, 0);

return false;

}

String responseBody = http.getString();

Serial.print("HTTP response code: ");

Serial.println(responseCode);

Serial.print("Server response: ");

Serial.println(responseBody);

http.end();

if (responseCode >= 200 && responseCode < 300) {

setLed(0, 20, 0);

return true;

}

setLed(20, 10, 0); // Orange

return false;

}

void printReading(const SensorReading& reading) {

Serial.println();

Serial.println("Reading summary");

Serial.print("Source: ");

Serial.println(reading.source);

Serial.print("Simulated: ");

Serial.println(reading.simulated ? "yes" : "no");

if (reading.valid) {

Serial.print("Temperature: ");

Serial.print(reading.temperature, 2);

Serial.println(" C");

Serial.print("Humidity: ");

Serial.print(reading.humidity, 2);

Serial.println(" %");

}

if (reading.error.length() > 0) {

Serial.print("Sensor error: ");

Serial.println(reading.error);

}

}

void collectAndSend() {

setLed(20, 20, 0); // Yellow

SensorReading reading = collectReading();

printReading(reading);

postReading(reading);

}

void setup() {

Serial.begin(115200);

delay(2000);

Serial.println();

Serial.println("ESP32-S3 DHT22 IoT client");

randomSeed(esp_random());

dht.begin();

connectToWiFi();

// DHT22 requires time to stabilize after power-up.

delay(3000);

collectAndSend();

previousReadingTime = millis();

}

void loop() {

if (millis() - previousReadingTime >= READING_INTERVAL_MS) {

previousReadingTime = millis();

collectAndSend();

}

}

# Appendix B Node Server

File: server/server.js

"use strict";

const express = require("express");

const Database = require("better-sqlite3");

const path = require("path");

const app = express();

const database = new Database("sensor-data.db");

const PORT = Number(process.env.PORT) || 3000;

const eventClients = new Set();

app.disable("x-powered-by");

app.use(express.json({ limit: "10kb" }));

app.use(express.static(path.join(__dirname, "public")));

database.pragma("journal_mode = WAL");

database.exec(`

CREATE TABLE IF NOT EXISTS sensor_readings (

id INTEGER PRIMARY KEY AUTOINCREMENT,

device_id TEXT NOT NULL,

crop TEXT,

reading_number INTEGER,

device_uptime_ms INTEGER,

air_temperature_c REAL NOT NULL,

air_humidity_percent REAL NOT NULL,

simulated INTEGER NOT NULL DEFAULT 0,

source TEXT,

sensor_error TEXT,

received_at TEXT NOT NULL DEFAULT CURRENT_TIMESTAMP

)

`);

const existingColumns = new Set(

database

.prepare("PRAGMA table_info(sensor_readings)")

.all()

.map(column => column.name)

);

if (!existingColumns.has("simulated")) {

database.exec(`

ALTER TABLE sensor_readings

ADD COLUMN simulated INTEGER NOT NULL DEFAULT 0

`);

}

if (!existingColumns.has("source")) {

database.exec(`

ALTER TABLE sensor_readings

ADD COLUMN source TEXT

`);

}

if (!existingColumns.has("sensor_error")) {

database.exec(`

ALTER TABLE sensor_readings

ADD COLUMN sensor_error TEXT

`);

}

const insertReading = database.prepare(`

INSERT INTO sensor_readings (

device_id,

crop,

reading_number,

device_uptime_ms,

air_temperature_c,

air_humidity_percent,

simulated,

source,

sensor_error

)

VALUES (?, ?, ?, ?, ?, ?, ?, ?, ?)

`);

const getReadingById = database.prepare(`

SELECT *

FROM sensor_readings

WHERE id = ?

`);

function broadcastReading(reading) {

const message =

`event: reading\n` +

`data: ${JSON.stringify(reading)}\n\n`;

for (const client of eventClients) {

client.write(message);

}

}

app.post("/api/readings", (request, response) => {

try {

const data = request.body;

const temperature = Number(data.air_temperature_c);

const humidity = Number(data.air_humidity_percent);

if (

typeof data.device_id !== "string" ||

data.device_id.trim() === "" ||

!Number.isFinite(temperature) ||

!Number.isFinite(humidity)

) {

return response.status(400).json({

success: false,

message: "Invalid or missing sensor values"

});

}

if (

temperature < -40 ||

temperature > 80 ||

humidity < 0 ||

humidity > 100

) {

return response.status(400).json({

success: false,

message: "Sensor value outside expected range"

});

}

const simulated =

data.simulated === true ||

data.simulated === 1 ||

data.simulated === "true"

? 1

: 0;

const result = insertReading.run(

data.device_id.trim(),

data.crop ?? null,

data.reading_number ?? null,

data.uptime_ms ?? null,

temperature,

humidity,

simulated,

data.source ?? null,

data.sensor_error ?? null

);

const storedReading = getReadingById.get(result.lastInsertRowid);

console.log("Reading stored:", storedReading);

broadcastReading(storedReading);

return response.status(201).json({

success: true,

message: "Reading stored",

reading: storedReading

});

} catch (error) {

console.error("Failed to store reading:", error);

return response.status(500).json({

success: false,

message: "Database error"

});

}

});

app.get("/api/readings", (request, response) => {

let limit = Number(request.query.limit);

if (!Number.isInteger(limit)) {

limit = 100;

}

limit = Math.max(1, Math.min(limit, 500));

const readings = database.prepare(`

SELECT *

FROM sensor_readings

ORDER BY id DESC

LIMIT ?

`).all(limit);

response.json(readings);

});

app.get("/api/events", (request, response) => {

response.set({

"Content-Type": "text/event-stream",

"Cache-Control": "no-cache, no-transform",

"Connection": "keep-alive",

"X-Accel-Buffering": "no"

});

response.flushHeaders();

response.write("retry: 3000\n\n");

eventClients.add(response);

console.log(`Dashboard connected. Active clients: ${eventClients.size}`);

request.on("close", () => {

eventClients.delete(response);

console.log(`Dashboard disconnected. Active clients: ${eventClients.size}`);

});

});

app.get("/api/health", (request, response) => {

response.json({

status: "healthy",

dashboard_clients: eventClients.size,

server_time: new Date().toISOString()

});

});

setInterval(() => {

for (const client of eventClients) {

client.write(": keep-alive\n\n");

}

}, 20000);

app.listen(PORT, "0.0.0.0", () => {

console.log(`Sensor server running on port ${PORT}`);

console.log(`Dashboard: http://localhost:${PORT}`);

});

# Appendix C Dashboard

File: server/public/index.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>ESP32-S3 Sensor Dashboard</title>

<script src="https://cdn.jsdelivr.net/npm/chart.js"></script>

<style>

* { box-sizing: border-box; }

body { margin: 0; padding: 24px; font-family: Arial, sans-serif; background: #f3f4f6; color: #111827; }

.dashboard { max-width: 1200px; margin: auto; }

header { display: flex; justify-content: space-between; align-items: center; gap: 15px; margin-bottom: 24px; }

h1 { margin: 0; font-size: 28px; }

.connection { padding: 8px 14px; border-radius: 20px; font-weight: bold; background: #fef3c7; color: #92400e; }

.connection.connected { background: #dcfce7; color: #166534; }

.connection.disconnected { background: #fee2e2; color: #991b1b; }

.cards { display: grid; grid-template-columns: repeat(auto-fit, minmax(180px, 1fr)); gap: 16px; margin-bottom: 20px; }

.card, .chart-card, .table-card { background: white; padding: 20px; border-radius: 12px; box-shadow: 0 2px 8px rgb(0 0 0 / 8%); }

.chart-card, .table-card { margin-bottom: 20px; }

.card-label { color: #6b7280; font-size: 14px; margin-bottom: 8px; }

.card-value { font-size: 27px; font-weight: bold; }

.small-value { font-size: 17px; overflow-wrap: anywhere; }

.chart-container { position: relative; height: 430px; }

.badge { display: inline-block; padding: 5px 10px; border-radius: 15px; font-size: 13px; font-weight: bold; }

.badge.real { color: #166534; background: #dcfce7; }

.badge.simulated { color: #92400e; background: #fef3c7; }

.table-wrapper { overflow-x: auto; }

table { width: 100%; border-collapse: collapse; }

th, td { padding: 11px; text-align: left; border-bottom: 1px solid #e5e7eb; white-space: nowrap; }

th { color: #374151; background: #f9fafb; }

@media (max-width: 600px) {

body { padding: 12px; }

header { align-items: flex-start; flex-direction: column; }

.chart-container { height: 350px; }

}

</style>

</head>

<body>

<main class="dashboard">

<header>

<div>

<h1>ESP32-S3 Sensor Dashboard</h1>

<p>Fenugreek microgreens monitoring</p>

</div>

<div id="connectionStatus" class="connection">Connecting</div>

</header>

<section class="cards">

<div class="card"><div class="card-label">Temperature</div><div class="card-value" id="temperature">-- C</div></div>

<div class="card"><div class="card-label">Humidity</div><div class="card-value" id="humidity">-- %</div></div>

<div class="card"><div class="card-label">Data type</div><div id="dataType"><span class="badge">Waiting</span></div></div>

<div class="card"><div class="card-label">Device</div><div class="card-value small-value" id="deviceId">--</div></div>

<div class="card"><div class="card-label">Source</div><div class="card-value small-value" id="source">--</div></div>

<div class="card"><div class="card-label">Last received</div><div class="card-value small-value" id="receivedAt">--</div></div>

</section>

<section class="chart-card">

<h2>Real-time Temperature and Humidity</h2>

<div class="chart-container"><canvas id="sensorChart"></canvas></div>

</section>

<section class="table-card">

<h2>Recent Readings</h2>

<div class="table-wrapper">

<table>

<thead><tr><th>ID</th><th>Time</th><th>Temperature</th><th>Humidity</th><th>Source</th><th>Type</th></tr></thead>

<tbody id="readingsTable"></tbody>

</table>

</div>

</section>

</main>

<script>

const MAX_CHART_POINTS = 60;

const MAX_TABLE_ROWS = 10;

const seenReadingIds = new Set();

const recentReadings = [];

const chart = new Chart(document.getElementById("sensorChart"), {

type: "line",

data: {

labels: [],

datasets: [

{

label: "Temperature (C)", data: [], borderColor: "#dc2626",

backgroundColor: "rgb(220 38 38 / 15%)", yAxisID: "temperatureAxis",

borderWidth: 2, pointRadius: 3, tension: 0.2

},

{

label: "Humidity (%)", data: [], borderColor: "#2563eb",

backgroundColor: "rgb(37 99 235 / 15%)", yAxisID: "humidityAxis",

borderWidth: 2, pointRadius: 3, tension: 0.2

}

]

},

options: {

responsive: true, maintainAspectRatio: false,

animation: { duration: 250 },

interaction: { mode: "index", intersect: false },

plugins: { legend: { position: "top" } },

scales: {

x: { title: { display: true, text: "Time" } },

temperatureAxis: { type: "linear", position: "left", title: { display: true, text: "Temperature (C)" } },

humidityAxis: { type: "linear", position: "right", min: 0, max: 100, title: { display: true, text: "Humidity (%)" }, grid: { drawOnChartArea: false } }

}

}

});

function parseDatabaseTime(value) {

if (!value) return new Date();

const isoValue = value.includes("T") ? value : value.replace(" ", "T") + "Z";

const date = new Date(isoValue);

return Number.isNaN(date.getTime()) ? new Date() : date;

}

function formatTime(value) { return parseDatabaseTime(value).toLocaleTimeString(); }

function formatDateTime(value) { return parseDatabaseTime(value).toLocaleString(); }

function updateCards(reading) {

document.getElementById("temperature").textContent = `${Number(reading.air_temperature_c).toFixed(2)} C`;

document.getElementById("humidity").textContent = `${Number(reading.air_humidity_percent).toFixed(2)} %`;

document.getElementById("deviceId").textContent = reading.device_id ?? "--";

document.getElementById("source").textContent = reading.source ?? "unknown";

document.getElementById("receivedAt").textContent = formatDateTime(reading.received_at);

const simulated = Number(reading.simulated) === 1;

document.getElementById("dataType").innerHTML = simulated

? '<span class="badge simulated">Simulated</span>'

: '<span class="badge real">Actual sensor</span>';

}

function updateChart(reading) {

chart.data.labels.push(formatTime(reading.received_at));

chart.data.datasets[0].data.push(Number(reading.air_temperature_c));

chart.data.datasets[1].data.push(Number(reading.air_humidity_percent));

while (chart.data.labels.length > MAX_CHART_POINTS) {

chart.data.labels.shift();

chart.data.datasets[0].data.shift();

chart.data.datasets[1].data.shift();

}

chart.update("none");

}

function renderTable() {

const table = document.getElementById("readingsTable");

table.replaceChildren();

for (const reading of recentReadings) {

const row = document.createElement("tr");

const values = [

reading.id,

formatDateTime(reading.received_at),

`${Number(reading.air_temperature_c).toFixed(2)} C`,

`${Number(reading.air_humidity_percent).toFixed(2)} %`,

reading.source ?? "unknown",

Number(reading.simulated) === 1 ? "Simulated" : "Actual"

];

for (const value of values) {

const cell = document.createElement("td");

cell.textContent = value;

row.appendChild(cell);

}

table.appendChild(row);

}

}

function addReading(reading) {

if (seenReadingIds.has(reading.id)) return;

seenReadingIds.add(reading.id);

updateCards(reading);

updateChart(reading);

recentReadings.unshift(reading);

if (recentReadings.length > MAX_TABLE_ROWS) recentReadings.pop();

renderTable();

}

async function loadHistoricalReadings() {

try {

const response = await fetch(`/api/readings?limit=${MAX_CHART_POINTS}`);

if (!response.ok) throw new Error(`History request failed: ${response.status}`);

const readings = await response.json();

readings.reverse().forEach(addReading);

} catch (error) {

console.error(error);

}

}

function startLiveStream() {

const status = document.getElementById("connectionStatus");

const events = new EventSource("/api/events");

events.onopen = () => {

status.textContent = "Live";

status.className = "connection connected";

};

events.addEventListener("reading", event => {

try { addReading(JSON.parse(event.data)); }

catch (error) { console.error("Invalid live reading:", error); }

});

events.onerror = () => {

status.textContent = "Reconnecting";

status.className = "connection disconnected";

};

}

async function initializeDashboard() {

await loadHistoricalReadings();

startLiveStream();

}

initializeDashboard();

</script>

</body>

</html>